浙江大学2023–2024学年秋冬学期

《计算机组成与设计》课程期末考试试卷

课程号： 111C0130 ，开课学院： 信息与电子工程学院

考试试卷： A卷、B卷、√C卷（请在选定项上打√）

考试形式：√闭、开卷（请在选定项上打√），

允许带 1 张A4纸大小的手写资料和计算器入场

考试日期： 2024 年 1 月 16 日, 考试时间： 120 分钟

诚信考试，沉着应考，杜绝违纪。

考生姓名：_________学号：_______________所属院系（专业）：____________

| 题序 | 一 | 二 | 三 | 四 | 五 | 总 分 |
| --- | --- | --- | --- | --- | --- | --- |
| 得分 |  |  |  |  |  |  |
| 评卷人 |  |  |  |  |  |  |

I. CHOICE (Only one correct answer, 60 points)

1. Assume that a virtual memory address is 32bit in length and the physical memory is 64MB.  What’s the size of a page table if page size is 32KB?  ( D )

A: 32KB          B: 64KB            C: 128KB         D: 512KB

2. Consider the following C code:

typedef unsigned char *pointer; // sizeof(unsigned char) = 1 byte

void show_bytes(pointer start, size_t len) {

for (int i = 0; i < len; i++)

printf("0x%x\n", start[i]);

}

int main() {

int a = 0x44332211;

show_bytes((pointer) &a, sizeof(int));

}

If this C code runs on a little-endian machine, what will we get on the third line of the terminal output? () C

A. 0x11

B. 0x22

C. 0x33

D. 0x44

3. A direct-mapped cache has 1K entries, cache line size is 2 words. What’s the capacity of the cache? ( C )

A: 2KB  		 B: 4KB   			C:  8KB  				 D: 16KB

4. For a given function, which programming language likely takes the most lines of

code?()C

A. C

B. Java

C. RISC-V assembly language

D. Python

5. Assume that integer is stored as a word (4 bytes) in memory, and -2 is stored at Memory address start from 0x00000004, so what Byte is store in memory address 0x00000007 in RISC-V computer? ( A )

A. 11111110  B. 11111111  C. 10000000  D. 00000010

6. Which operations can isolate a field in a word?()A

A. A shift left followed by a shift right

B. AND

C. ADD

D. SUB

7. MIMD stands for ____. A

A. Multiple instruction multiple data

B. Multiple instruction memory data

C. Memory instruction multiple data

D. Multiple information memory data

8. Which of the following statements must be true about compilers?()A

A. Compiled code generally is only able to run on one ISA.

B. The code produced is always more efficient and higher performance than that produced by interpreters.

C. There is only one compiler per language.

D. Compilers are always more difficult to write than interpreters.

9. The idea of cache memory is based on _____. A

A. the property of locality of reference

B. the heuristic 90-10 rule

C. the fact that reference generally tend to cluster

D. all of the above

10. The execution of the following two instructions may have the _____.()A

add x5,x3,x4

sd x5,0(x6)

A. RAW (Read after Write)					B. WAW (Write after Write)

C. WAR (Write after Read)					D. No data dependency

11. Suppose there was a 16-bit IEEE 754 floating-point format with five exponent bits. What would

be the likely range of numbers it could represent? (C )

A. 1.0000 0000 00×20 to 1.1111 1111 11×231,0

B. ±1.0000 0000 0×2-14 to ±1.1111 1111 1×215, ±0, ±∞, NaN

C. ±1.0000 0000 00×2-14 to ±1.1111 1111 11×215, ±0, ±∞, NaN

D. ±1.0000 0000 00×2-15 to ±1.1111 1111 11×214, ±0, ±∞, NaN

12. Given the following RISC-V assembly code (and assuming all registers start at 0):

addi 	t1, x0, 10

add 	t2, t1, t1

repeat:	addi 	t2, t2, -4

add 	t3, t2, t2

addi 	t1, t1, -2

bne 	x0, t1, repeat

What is the final value of register t3? A

A.0

B.-4

C.8

D.4

13. Assume the current PC is 0x10000010, what’s next value of PC after execution of “jal x0, 100”? ( C )

A.0x00000064  B. 0x10000014  C. 0x10000074  D. 0x10000064

14. For a 32-bit cache-memory system, a 64KB, 2-way set-associative cache has 4 words cache line size, how many bits are there in such cache’s tag?()A

A. 17			B. 19			C. 21			D. 23

15. Assuming that there are 100 processors, what is the proportion of parallel part in parallel software to achieve a 80 speedup? (A)

A：99.75% 			B：99.15%			C：98.75%				D：98.15%

16. A page fault occurs when ______. C

A. there is an error in a specific page

B. a program accesses a page of main memory

C. a program accesses a page not currently in main memory

D. a program accesses a page belonging to another program

17. When a subroutine is called, the address of the instruction following the CALL instruction stored in/on the__. D

A. stack pointer

B. accumulator

C. program counter

D. stack

17. Which one of three categories occurs in a set-associative or direct-mapped cache when multiple blocks compete for the same set and that are eliminated in a fully associative cache of the same size? (C)

A：compulsory misses 	 	B：capacity misses			C：conflict misses

19. Assum the initial value in x10 is 19. What is the value of x10 for the following sequence of RISC-V instructions?  C

slli x10, x10, 2

xori x10, x10, 0xfff

A. 76			B.-76			C.-77			D.179

20. Let A be a 1024×1024 matrix of 32-bit int elements stored in row-major order, aligned to

the beginning of a cache line.

for (int i = 1; i < 16; i++) {

int x = A[0][i-1];

int y = A[i][i];

A[i][i] = x + y;

}

Assume that memory accesses are executed in the order shown in the program – i.e., the

compiler does not reorder load and store instructions. Variables x, y, and i are held in registers.

Consider a 4 KiB direct-mapped L1 data cache with 16-byte cache lines. Assume that the cache is initially empty. The numbers of cache hits on the loop shown above is ___. B

A.16

B.15

C.14

D.0

II. TRUE OR FALSE (10 points)

1.___ The LRU can be improved by providing a little randomness in the access. T

2.___ High associativity in a cache reduces compulsory misses. F

3.___ In the memory hierarchy, as the speed of operation increases the memory size also increases. F

4.___ Cache performance is of less importance in faster processors because the processor speed compensates for the high memory access time. F

5.___ A write-through cache typically requires less bus bandwidth than a write-back cache. F

6.___ The direct-mapped cache has largest tag overhead. F

7.___ The CPI of superscalar processors can be less than one. T

8.___ Cache and main memory are made of SRAM and DRAM respectively.T

9.___ Cache block size (B) can only affect miss latency. F

10.___ We use large cache line due to temporal locality. F

III. Assembly and Pipeline (10 points)

Consider the following sequence of instructions:

| add lw lw or sw | x15, x12, x11 x13, 8(x15) x12, 0(x2) x13, x15, x13 x13, 0(x15) |
| --- | --- |

Assume each datapath stage requires the following amount of time to complete:

- Instruction fetch (IF): 30 ns

- Instruction decode (ID): 20 ns

- Execute / address calculation (EX): 25 ns

- Memory access (MEM): 30 ns

- Register write back (WB): 20 ns

a) How long will this sequence of instructions take in a single-cycle datapath?  ___625____ns  (2 points)

b) If we assume a pipelined datapath without forwarding, how long will one loop iteration take in a pipelined datapath? ___420____ns (2 points)

c) If we now assume a pipelined datapath with forwarding, how many data hazard(s) cannot be solved with forwarding? (number only)

__0____ (3 points)

d) If we now assume a pipelined datapath with forwarding, how long will one iteration take?

__270_____ns (3 points)

IV. CACHE (10 points)

For all the questions in this problem, assume that we are using a 12-bit machine with a byte-addressable memory and a direct-mapped cache. The cache can hold up to 16 cache lines.

1.How many bits do you need for the set index? (2pts) 4

_____

2. The following sequence of 9 memory accesses generates the hits/misses shown. Some miss/hit entries are intentionally left blank. The cache is initially empty. Note that the addresses are written in binary with spaces added between each 4 bits for readability. These are not necessarily the tag/index/offset boundaries.

| # | Address | Hit/Miss |
| --- | --- | --- |
| 1 | 1101 1111 0000 | Miss |
| 2 | 0000 1101 1111 | Miss |
| 3 | 1101 0111 0101 | Miss |
| 4 | 0000 1101 1100 | Hit |
| 5 | 1101 1111 0011 | Miss |
| 6 | 1111 0111 0010 | Miss |
| 7 | 1101 1111 0000 | Miss |
| 8 | 0000 1101 1101 | Hit |
| 9 | 1111 0111 0100 | Miss |

2.1 What is the number of tag bits? (2pts) 5

_____

2.2 What is the number of offset bits? (2pts) 3

_____

2.3 Fill the miss/hit for each of the blank entries. (4pts)

V. VIRTUAL MEMORY (10 points)

Assume the following: a 32-bit virtual address space, with a 1KB page size

1. How many bits are in the offset portion of the virtual address? (2 points)
_____ 10

2. How many bits are in the VPN portion of the virtual address? (2 points)

_____ 22

Now, let’s focus on the page table. Assume each page table entry is 4 bytes in size. Assuming a linear page table:

3. How many entries are in the table? (2 points)

_____ 2^22

4. What is the total size of the table? (2 points)

_____MB 16

5. In a live system, how much memory would be occupied by page tables if there are 100 processes? (2 points)s

_____MB 1600