浙江大学2024–2025学年秋冬学期

《计算机组成与设计》课程期末考试试卷

课程号： 67190020 ，开课学院： 信息与电子工程学院

考试试卷： √A卷、B卷（请在选定项上打√）

考试形式：√闭、开卷（请在选定项上打√），

允许带 1 张A4纸大小的手写资料和计算器入场

考试日期： 2024 年 1 月 2 日, 考试时间： 120 分钟

诚信考试，沉着应考，杜绝违纪。

考生姓名：_________学号：_______________所属院系（专业）：____________

| 题序 | 一 | 二 | 三 | 四 | 五 | 总 分 |
| --- | --- | --- | --- | --- | --- | --- |
| 得分 |  |  |  |  |  |  |
| 评卷人 |  |  |  |  |  |  |

I. CHOICE (Only one correct answer, 60 points)

1. The DRAMs are basically used as ______ (   )  C

A. register

B. cache

C. main memory

D. disk

2. What is the decimal value of this 32-bit two’s complement number? (   ) A

11111111 11111111 11111111 11111100

A: -4    		 B: -16    			C:  4,294,967,292 		 D: -8

3. Assume x5 holds the value 0x0000000001010000. What is the value of x6 after the following instructions? (   ) B

bge x5, x0, ELSE

add x6, x5,x0

jal x0, DONE

ELSE:  ori x6, x0, 2

DONE:

A: 80			B: 2					C: 160					D: 0

4. Consider the following code:

lb x6, 0(x7)

sb x6, 8(x7)

Assume that the register x7 contains the address 0×10000000 and the data at

address is 0×1122334455667788.

- What value is stored in 0×10000008 on a big-endian machine?(   ) A

A: 0x11			B: 0x22				C: 0x77					D:0x88

- What value is stored in 0×10000008 on a big-endian machine?(   ) D

A: 0x11			B: 0x22				C: 0x77					D:0x88

5. What decimal number does the bit pattern 0xAC000000 represent if it is a two’s complement integer?(   ) C

A: 201326592		B: 2885681152		C: -1409286144			D: -201326592

6. What is the minimum number of cycles needed to completely execute n instructions on a CPU with a k stage pipeline?(   ) B

A: n+k				B: n+k-1			C: n*k					D:n-k

7. Assume that x11 is initialized to 11 and x12 is initialized to 22. Suppose you executed the code below on a version of the pipeline introduced in our class that does not handle data hazards (i.e., the programmer is responsible for addressing data hazards by inserting NOP instructions where necessary). What would the final values of registers  x14 be?(   ) A

addi x11, x12, 5

add x11, x11, x12

addi x14, x11, 15

A: 36				B:42				C: 53					D:33

8. How many NOP instructions should be added to the code below so that it will run correctly on a pipeline that does not handle data hazards?(   ) C

addi x11, x12, 5

add x13, x11, x12

addi x14, x11, 15

add x15, x13, x12

A: 1				B: 2					C: 3						D: 4

9. Dividing 101012 by 112, the quotient is ___and the reminder is___ . C

A. 1002 012

B. 1112 012

C. 1112 02

D. 1002 02

10. A direct-mapped cache has 1K entries, cache line size is 2 words. What’s the capacity of the cache? (   ) C

A: 2KB  		 B: 4KB   			C:  8KB  				 D: 16KB

11. Cache acts between ______. (   ) A

A. CPU and RAM

B. RAM and ROM

C. CPU and Hard disk

D. none of these

12. Which of the following cache designer guideline is not valid? (   ) B

A. Fully associative caches have no conflict misses.

B. In reducing misses, associativity is more important than capacity.

C. The higher the memory bandwidth, the larger the cache block.

D. The shorter the memory latency, the smaller the cache block.

13. Assume the current PC is 0x10000010, what’s next value of PC after execution of “jal x0, 100”? (   ) C

A.0x00000064  B. 0x10000014  C. 0x10000074  D. 0x10000064

14. Assuming that there are 100 processors, what is the proportion of parallel part in parallel software to achieve a 80 speedup? (   ) A

A：99.75% 			B：99.15%			C：98.75%				D：98.15%

15. Which one of three categories occurs in a set-associative or direct-mapped cache when multiple blocks compete for the same set and that are eliminated in a fully associative cache of the same size? (   ) C

A：compulsory misses 	 	B：capacity misses			C：conflict misses

16. The fastest data access is provided using (   ) D

A. Caches  				B. DRAM’s  		C. SRAM’s  			D.Registers

17. Which operations can isolate a field in a word?(   )A

A. A shift left followed by a shift right

B. AND

C. ADD

D. SUB

18. Assuming forgetting to implement the hazard detection unit, how many cycles does it take to execute the following instruction sequence on a 5-stage pipeline with CPI 1? (   ) C

I1:  ld 	x1,40(x6)

I2: 	add 	x2,x3,x1

I3: 	ld 	x1,16(x2)

I4: 	sd 	x2,20(x4)

I5: 	and 	x1,x1,x4

A, 8 					B：7				C：6					D：5

19. The idea of cache memory is based on _____. (   ) A

A. the property of locality of reference

B. the heuristic 90-10 rule

C. the fact that reference generally tend to cluster

D. all of the above

II. TRUE OR FALSE (10 points)

1.(   ) A main memory access typically consumes less energy than a register file access.F

2. (   ) The memory hierarchy of a typical computer system comprises different memory technologies. T

3. (   ) A program with no branches has high temporal locality in its instruction memory references. F

4. (   ) A cache that has a block size equal to word size of memory access instructions cannot exploit spatial locality. T

5. (   ) The entire page table is typically stored in physical memory. F

6. (   ) An executable binary file that can run on a RISC-V CPU may not be able to execute on another RISC-V CPU. T

7. (   ) The CPI of superscalar processors can be more than one. T

8. (   ) Virtual-to-physical address translation is on the critical path of a memory access. T

9. (   ) Cache block size (B) can only affect miss latency. F

10. (   ) There is no way to reduce compulsory misses. F

III. Assembly and Pipeline (10 points)

Consider the fragment of RISC-V assembly below:

sd x29, 12(x16)

ld x29, 8(x16)

sub x17, x15, x14

beqz x17, label

add x15, x11, x14

sub x15, x30, x14

Suppose we modify the pipeline so that it has only one memory (that handles both instructions and data). In this case, there will be a structural hazard every time a program needs to fetch an instruction during the same cycle in which another instruction accesses data.

1) In general, is it possible to reduce the number of stalls/NOPs resulting from this structural hazard by reordering code?[3 points]

No

eordering code won’t help. Every instruction must be fetched; thus, every data access causes a stall. Reordering code will just change the pair of instructions that are in conflict.

2) We have seen that data hazards can be eliminated by adding NOPs to the code. Can you do the same with this structural hazard? [3 points]

No

ou can’t solve this structural hazard with NOPs, because even the NOPs must be fetched from instruction memory.

3) Approximately how many percentage stalls would you expect this structural hazard to generate in a typical program? (Use the instruction mix below) [4 points]

| R-type/I-type (non-ld) | ld | sd | Beq |
| --- | --- | --- | --- |
| 52% | 25% | 11% | 12% |

36%

very data access will cause a stall

IV. CACHE (10 points)

You are trying to reverse-engineer the characteristics of a cache in a system, so that you can design a more e-cient, machine-specic implementation of an algorithm you are working on. To do so, you have come up with three sequences of memory accesses to various bytes in the system in an attempt to determine the following four cache characteristics:

- Cache block size (8, 16, 32, 64, or 128 B).

- Cache associativity (2-, 4-, or 8-way).

- Cache replacement policy (LRU or FIFO)

The only statistic that you can collect on this system is cache hit rate after performing each sequence of memory accesses. Here is what you observe:

| Sequence | Addresses Accessed (Oldest → Youngest) | Hit Rate |
| --- | --- | --- |
| 1. | 0→16→24→25→1024→255→1100→305 | 2/8 |
| 2. | 31→65536→65537→131072→262144→8→305→1060 | 3/8 |
| 3. | 262145→65536→4 | 2/3 |

Assume that the cache is initially empty at the beginning of the first sequence, but not at the beginning of the second and third sequence. The sequences are executed back-to-back, i.e., no other accesses take place in between sequences. Thus, at the beginning of the second sequence, the contents are the same as at the end of the first sequence. At the beginning of the third sequence, the contents are the same as at the end of the second sequence.

Based on what you observe, what are the following characteristics of the cache?

- [4 points] Cache block size (8, 16, 32, 64, or 128 B)? 					(     )

- [3 points] Cache associativity (2-, 4-, or 8-way)?							(     )

- [3 points] Cache replacement policy (LRU or FIFO)?					(     )

Answer:

1)

16 B.

Explanation:

Cache hit rate is 2/8 in sequence 1. This means that there are 2 hits. Depending on the cache block size, we can group addresses that belong to the same cache block as follows:

- 8B: {0}, {16}, {24,25}, {255}, {305}, {1024}, {1100}. ∴ Number of possible hits = 1

- 16B: {0}, {16,24,25}, {255}, {305}, {1024}, {1100}. ∴ Number of possible hits = 2

- 32B: {0,16,24,25}, {255}, {305}, {1024}, {1100}. ∴ Number of possible hits = 3

- 64B: {0,16,24,25}, {255,305}, {1024}, {1100}. ∴ Number of possible hits = 4

- 128B: {0,16,24,25}, {255,305}, {1024,1100}. ∴ Number of possible hits = 5

Therefore, we can know that the cache block size is 16B.

2)

2-way

Explanation:

Cache hit rate is 3/8 in sequence 2, which means that there are 3 hits.

We already know that the cache block size is 16B. Thus, there are 4 offset bits.

The access to address 31 in sequence 2 would hit because the cache block would not be replaced.

The access to address 305 in sequence 2 would hit because the cache block would not be replaced.

The access to address 65537 in sequence 2 would hit because the cache block would not be replaced.

Therefore, all the other accesses should miss.

The access to address 65536, 131072 and 262144 in sequence 2 would miss because addresses 65536, 131072 and 262144 do not belong to any cache block previously accessed. Addresses 65536, 131072 and 262144 would be placed in set 0 if the cache associativity is 2-way, 4-way, or 8-way, independently of the cache size.

Address 8 must be a miss, so its cache block must be replaced by cache blocks that map to set 0 (addresses 65536, 131072 and 262144). For this to happen, the associativity must be 2-way.

Therefore, the cache is 2-way associative.

FIFO.

From questions (a) and (b), we already know the following facts:

- The cache block size is 16 B.

- The cache is 2-way.

Cache hit rate is 2/3 in sequence 3, which means that there are 2 hits.

With the LRU policy only the access to address 262145 in sequence 3 would hit. With

the FIFO policy, accesses to addresses 262145 and 4 in sequence 3 would hit.

Therefore, the cache adopts the FIFO policy.

五．VIRTUAL MEMORY  (10 points)

Assume the following: a 32-bit virtual address space, with a 1KB page size

1. How many bits are in the offset portion of the virtual address? [2 points]
_____ 10

2. How many bits are in the VPN portion of the virtual address? [2 points)

_____ 22

Now, let’s focus on the page table. Assume each page table entry is 4 bytes in size. Assuming a linear page table:

3. How many entries are in the table? [2 points)

_____ 2^22

4. What is the total size of the table? [2 points]

_____MB 16

5. In a live system, how much memory would be occupied by page tables if there are 100 processes? [2 points)

_____MB 1600