Homework 3

Problem1

In this exercise, we examine how pipelining affects the clock cycle time of the processor. Problems in this exercise assume that individual stages of the datapath have the following latencies:

| IF | ID | EX | MEM | WB |
| --- | --- | --- | --- | --- |
| 250ps | 400ps | 150ps | 350ps | 200ps |

Also, assume that instructions executed by the processor are broken down as follows:

| alu | beq | lw | sw |
| --- | --- | --- | --- |
| 40% | 20% | 25% | 15% |

- What is the clock cycle time in a pipelined and non-pipelined processor?

- What is the total latency of an LW instruction in a pipelined and non-pipelined processor?

- If we can split one stage of the pipelined datapath into two new stages, each with half the latency of the original stage, which stage would you split and what is the new clock cycle time of the processor?

Problem 2

Look for data hazards in the code below, and figure out how forwarding could be used to solve them.

them.

| Instruction | C1 | C2 | C3 | C4 | C5 | C6 | C7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. addi t0, a0, -1 | IF | ID | EX | MEM | WB |  |  |
| 2. and s2, t0, a0 |  | IF | ID | EX | MEM | WB |  |
| 3. sltiu a0, t0, 5 |  |  | IF | ID | EX | MEM | WB |

Problem 3

This exercise is intended to help you understand the relationship between forwarding, hazard detection, and ISA design. Problems in this exercise refer to the following sequence of instructions, and assume that it is executed on a 5-stage pipelined datapath:

add x5, x2, x1

lw x3, 4(x5)

or x3, x5, x3

lw x2, 0(x2)

sw x3, 0(x5)

- if there is no forwarding or hazard detection, insert NOP to ensure correct execution.

- Repeat 1) but now use NOP only when a hazard cannot be avoided by changing or rearranging these instructions. You can assume register x7 can be used to hold temporary values in your modified cade.

- If the processor has forwarding, but we forget to implement the hazard detection unit, what happens when this code executes?

Problem 4