作业4

Design a uniform linear scanning array whose maximum of the array factor is 600 from the axis of the array (θ0 = 600). The desired half-power beamwidth is 130.

Determine

- The range of d for no grating lobes

- the excitation of the elements (amplitude and phase),

- number of array elements

- Chose d=0.5λ, us HFSS to simulate this array and calculate its Directivity (in dB), where the element can be dipole.

From the figure, θ0=600 & HPBW=130->Nd=5λ.

For no grating lobes, d<2/3λ.

Chose d=λ/2, so β=-900, N=5λ/d=10.