实验报告

姓名：     郑智瑄         专业：   电子科学与技术      学号：    3230103913

课程名称：     数字系统              任课老师：          李宇波

实验名称：                lab4                     实验日期：      2024/3/27

1 实验目的和要求

1.1 实验目的

学习如何使用FSM实现电子锁。

1.2 实验要求

用下面的功能图实现在课堂上学到的电子锁。

在实验报告中呈现以下结果：

1. prelab4文档要求中的测试结果

2.行为级代码、仿真以及板子上的结果

3.手写状态图，由状态转换表获得状态方程、驱动方程的过程，以及最后的状态机电路

4.根据手写的状态机电路写成的电路级代码

2 实验内容

- prelab4文档要求中的测试结果（prelab4附于文末）

- 按下按钮W19，LED灯U16亮，再次按下按钮，LED熄灭

- 行为级代码、仿真以及板子上的结果（lab4 behavior level代码，ab4_tb代码附于文末）

- 手写状态图，由状态转换表获得状态方程、驱动方程的过程，以及最后的状态机电路

4.根据手写的状态机电路写成的电路级代码（lab4 assign_implementation附于文末）

3 实验总结

- 要持之以恒的学习，不畏难不惧难

- 要学会多问，多与老师交流

4 源代码

//prelab4

module onoff_sync

(input clk,reset, button_in,

output reg light);

// synchronizer

reg button,btemp;

always @(posedge clk)

{button,btemp} <={btemp,button_in};

// debounce push button

wire bpressed;

debounceIndex db1(.reset(reset),.clk(clk),.noisy(button),.clean(bpressed));

reg old_bpressed;//state last clk cycle

always @(posedge clk)

begin

if (reset)

begin light <=0;old_bpressed <=0; end

else if(old_bpressed==0 && bpressed==1)

// button changed from 0 to 1

light <= ~light;

old_bpressed <= bpressed;

end

endmodule

module debounceIndex (reset, clk, noisy, clean);

input reset, clk, noisy;

output clean;

parameter NDELAY = 65000;

parameter NBITS = 20;

reg [NBITS-1:0] count;

reg xnew, clean;

always @(posedge clk)

if (reset)

begin

xnew <= noisy;

clean <= noisy;

count <= 0 ;

end

else if (noisy!=xnew)

begin

xnew <= noisy;

count <= 0;

end

else if (count == NDELAY)

clean <= noisy;

else

count <= count+1;

endmodule

//lab4 behavior level

module E_LOCK_FSM(

input clk,reset,b0_in,b1_in,

output out,

output [7:0] display

);

//synchronizer

reg b0_sync,b0temp,b1_sync,b1temp;

always @(posedge clk)

{b0_sync,b0temp} <={b0temp,b0_in};

always @(posedge clk)

{b1_sync,b1temp} <={b1temp,b1_in};

//debounce

wire b0_de,b1_de;

debounceIndex de1(.reset(reset), .clk(clk), .noisy(b0_sync), .clean(b0_de));

debounceIndex de2(.reset(reset), .clk(clk), .noisy(b1_sync), .clean(b1_de));

//conversion

reg b0,b1;

reg Q0,Q1;

always @(posedge clk) begin

Q0 <= b0_de;

b0 <= ~Q0&b0_de;

end

always @(posedge clk) begin

Q1 <= b1_de;

b1 <= ~Q1&b1_de;

end

parameter S_RESET = 0; parameter S_0 = 1;

parameter S_01 = 2; parameter S_010 = 3;

parameter S_0101 = 4; parameter S_01011 = 5;

reg [2:0] state ,next_state;

always@(*) begin

if(reset) next_state = S_RESET;

else case (state)

S_RESET: next_state = b0 ? S_0  :b1 ? S_RESET:state;

S_0:     next_state = b0 ? S_0  :b1 ? S_01   :state;

S_01:    next_state = b0 ? S_010:b1 ? S_RESET:state;

S_010:   next_state = b0 ? S_0  :b1 ? S_0101 :state;

S_0101:  next_state = b0 ? S_010:b1 ? S_01011:state;

S_01011: next_state = b0 ? S_0  :b1 ? S_RESET:state;

default: next_state = S_RESET;

endcase

end

always@(posedge clk) state<= next_state;

// LED

wire [6:0]segs;

LED L1(.I(state),.Segs(segs));

assign out = (state == S_01011);

assign display = {1'b0,segs};

endmodule

module debounceIndex (reset, clk, noisy, clean);

input reset, clk, noisy;

output clean;

parameter NDELAY = 65000;

parameter NBITS = 20;

reg [NBITS-1:0] count;

reg xnew, clean;

always @(posedge clk)

if (reset)

begin

xnew <= noisy;

clean <= noisy;

count <= 0 ;

end

else if (noisy!=xnew)

begin

xnew <= noisy;

count <= 0;

end

else if (count == NDELAY)

clean <= noisy;

else

count <= count+1;

endmodule

module LED(

input [2:0]I,

output reg [6:0]Segs

);

always @(I)begin

case (I)

0:Segs= 7'b0000001;

1:Segs= 7'b1001111;

2:Segs= 7'b0010010;

3:Segs= 7'b0000110;

4:Segs= 7'b1001100;

5:Segs= 7'b0100100;

6:Segs= 7'b0100000;

7:Segs= 7'b0001111;

default:Segs= 7'b0000000;

endcase

end

endmodule

//lab4_tb

module E_LOCK_tb();

reg clk,reset,b0_in,b1_in;

wire out;

wire [7:0] display;

E_LOCK_FSM E1(.clk(clk),.reset(reset),.b0_in(b0_in),.b1_in(b1_in),.out(out),.display(display));

initial begin

clk=0;

reset=1;

b0_in=0;

b1_in=0;

#100

reset=0;

end

always begin

#5

clk=~clk;

end

always begin

#100 b0_in=1;

#100 b0_in=0;

#100 b1_in=1;

#100 b1_in=0;

#100 b0_in=1;

#100 b0_in=0;

#100 b1_in=1;

#100 b1_in=0;

#100 b1_in=1;

#100 b1_in=0;

end

endmodule

//lab4 assign_implementation

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

module E_LOCK_FSM(

input clk,reset,b0_in,b1_in,

output reg out,

output reg [7:0] display,

output reg [2:0] next_state,

output reg b0,b1

);

//synchronizer

reg b0_sync,b0temp,b1_sync,b1temp;

always @(posedge clk)

{b0_sync,b0temp} <={b0temp,b0_in};

always @(posedge clk)

{b1_sync,b1temp} <={b1temp,b1_in};

//debounce

wire b0_de,b1_de;

debounceIndex de1(.reset(reset), .clk(clk), .noisy(b0_sync), .clean(b0_de));

debounceIndex de2(.reset(reset), .clk(clk), .noisy(b1_sync), .clean(b1_de));

//conversion

//  reg b0,b1;

reg Q0,Q1;

always @(posedge clk) begin

Q0 <= b0_de;

b0 <= ~Q0&b0_de;

end

always @(posedge clk) begin

Q1 <= b1_de;

b1 <= ~Q1&b1_de;

end

parameter S_RESET = 0; parameter S_0 = 1;

parameter S_01 = 2; parameter S_010 = 3;

parameter S_0101 = 4; parameter S_01011 = 5;

reg [2:0] state ;

//reg [2:0] next_state;

//Secondary logic circuit

always@(posedge clk) begin

if(reset) next_state =S_RESET;

else begin

//1000X+10X00+01101

next_state[2]= state[2]&~state[1]&~state[0]&~b0|

state[2]&~state[1]          &~b0&~b1|

~state[2]& state[1]& state[0]&~b0& b1;

//010X0+01X00+00101+10010

next_state[1]=~state[2]& state[1]&~state[0]&    ~b1|

~state[2]& state[1]          &~b0&~b1|

~state[2]&~state[1]& state[0]&~b0& b1|

state[2]&~state[1]&~state[0]& b0&~b1;

//0X1X0+101X0+10001+XXX10

next_state[0]=~state[2]& state[0]               &~b1|

state[2]&~state[1]& state[0]     &~b1|

state[2]&~state[1]&~state[0]&~b0& b1|

b0&~b1;

end

end

always@(posedge clk) state<= next_state;

// LED

always @(*) begin

out=state[2]&~state[1]&state[0];

display[7]=0;

display[6]=state[2]&~state[0]|~state[2]&~state[1]&state[0];

display[5]=state[2]&state[0];

display[4]=state[1]&~state[0];

display[3]=~state[2]&~state[1]&state[0]|state[2]&~state[0];

display[2]=~state[2]&state[0]|state[2]&~state[1];

display[1]=state[1]|~state[2]&state[0];

display[0]=~state[2]&~state[1];

end

endmodule

module debounceIndex (reset, clk, noisy, clean);

input reset, clk, noisy;

output clean;

parameter NDELAY = 1;

parameter NBITS = 20;

reg [NBITS-1:0] count;

reg xnew, clean;

always @(posedge clk)

if (reset)

begin

xnew <= noisy;

clean <= noisy;

count <= 0 ;

end

else if (noisy!=xnew)

begin

xnew <= noisy;

count <= 0;

end

else if (count == NDELAY)

clean <= noisy;

else

count <= count+1;

endmodule